Changing comm. types on IP terminals.

Omni Series:  To change from dial to IP and vice versa on these terminals, press the number six for quick set up, then choose communication, then choose view, and the first option is Nova Auth Comm.  To have the terminal to process IP choose comm. server and to have the terminal process through dial choose dial.  Also make sure the backup comm. is set correctly.  This field is also found in communication menu and is one parameter after auth comm.  The nova settle comm. will also need to be changed to match the nova auth comm. and is located in the communication menu and is two parameters below backup comm.

Hypercom T4100:  Function 60 and then Enter.  Choose disable Ethernet and enable PPP to set terminal to dial and then press enter to confirm.

Nurit 8320IP:  Press Menu/ESC, then the number 8.  Choose #2 for Main comm. and then press forced until you see dial to make the terminal use dial as the main option.

Ingenico 5100IP:  Press Admin then the number 3 for setup menu, then the number 1 for quick setup, then press next until you see comms mode on the display, choose edit and then choose which way you want the terminal to process either dial or IP.