SYSTEM MENU DIFFERENCES

| Old versions | 8A4 |
| --- | --- |
| SYS MODE MENU 1 CONTRAST F2 CLOCK F3 RESTART F4 | SYS MODE MENU 1 EDIT PARAMATERS F2 DOWNLOAD F3 RESTART F4 |
| SYS MODE MENU 2 DOWNLOAD F2 RAM FILES F3 FLASH FILES F4 | SYS MODE MENU 2 MEMORY FUNCTIONS F2 TERMINAL INFO F3 CLOCK F4 |
| SYS MODE MENU 3 CONFIG INFO F2 EDIT F3 PASSWORDS F4 | SYS MODE MENU 3 CONTRAST F2 PASSWORDS F3 IPP KEY LOAD F4 |
| SYS MODE MENU 4 REMOTE DIAGS F2 ERROR LOG F3 DEBUGGER F4 | DIAGNOSTIC FUNCTIONS ARE UNDER TERMINAL INFO [F3] ON SYSTEM MENU SCREEN 2 |
| SYS MODE MENU 5 SCREEN DIAG F2 KEYBOARD DIAG F3 MAG CARD DIAG F4 | SYS MODE MENU DIAGS AND LOGS F3 PRINTER F2 KEYBOARD DIAG F3 MAG CARD DIAG F4 SYS MODE DIAGS SCREEN DIAG F2 IPP DIAG F3 SMART CARD DIAG F4 SYS MODE DIAGS REMOTE DIAGS F2 DEBUGGER F3 ERROR LOG F2 TAMPER LOG F3 |
| SYS MODE MENU 6 IP DIAG F2 IPP KEY LOAD F3 PRINTER DIAG F4 | SYSTEM INFO - TERMINAL SERIAL NUMBER, PART NUMBER, OPSYS VERSION AND MODEL NUMBER ARE UNDER TERMINAL INFO [F3] ON SYSTEM MENU SCREEN 2 SYS MODE MENU SYSTEM INFO F2 SERNO PTID PART OS VER MODL |
| SYS MODE MENU 7 RAM DIRECTORY F3 FLASH DIRECTORY F4 | MEMORY FUNCTIONS ARE UNDER MEMORY FUNCTIONS [F2] ON SYSTEM MENU SCREEN 2 |
|  | SYS MODE MENU 2 MEMORY FUNCTIONS F2 TERMINAL INFO F3 CLOCK F4 Select [F2] for memory functions SYS MODE MEM FUNCS RAM: XXXX FLASH XXXX USAGE F2 DIRECTORIES F3 CLEAR MEM F4 Select [F2] for memory usage information MEMORY USAGE RAM FILES INUSE AVAIL FLASH FILES INUSE AVAIL |
|  | SYS MODE MEM FUNCS RAM: XXXX FLASH XXXX USAGE F2 DIRECTORIES F3 CLEAR MEM F4 Select [F3] for directories SYS MODE DIRECTORY GROUP ID: _1 RAM FILES F3 FLASH FILES F4 |
| SYS MODE MENU 8 ICC DIAG F2 SELECT DOWNLAOD UI F3 |  |

Clearing memory functions

| Old versions | 8A4 |
| --- | --- |
| SYS MODE MENU 2 DOWNLOAD F2 RAM FILES F3 FLASH FILES F4 Select [F3] to clear ram files SYS MODE RAM CLEAR GROUP 1 F2 CLEAR ALL FILES F3 Select the appropriate files to clear SYS MODE MENU 2 DOWNLOAD F2 RAM FILES F3 FLASH FILES F4 Select [F4] to clear FLASH files. SYS MODE FLASH CLEAR GROUP 1 F2 CLEAR ALL FLIES F3 DEFRAG F4 Select the appropriate files to clear | SYS MODE MEM FUNCS RAM: XXXX FLASH XXXX USAGE F2 DIRECTORIES F3 CLEAR MEM F4 Select [F4] to clear the memory SYS MODE MEMORY ENTER GROUP 1 If the GID displayed is correct select [ENTER] to clear the memory. If not, key in the correct GID and press [ENTER]. SYS MODE MEMORY CLEAR CONFIG.SYS F2 CLEAR GX FILES F3 CLEAR ALL GROUPS F4 Note – When clearing the memory, the terminal will not display a message that the memory has been cleared. The terminal will however return to the SYS MODE MENU FUNC SCREEN. |

# Full dial download

| Key | Value |
| --- | --- |
| *ZA | Application name |
| *ZT | Merchants NOVA number |
| *ZP | VeriCentre phone number |

| FULL DIAL DOWNLOAD |  |  |  |
| --- | --- | --- | --- |
| Step | Action | Display | Keys |
| 1 | Press indicated key on the keypad. | <Current Menu Display> | Press [F2] + [F4] simultaneously. |
| 2 | Press indicated key on the keypad. | SYSTEM MODE ENTRY PASSWORD | Key in 1 Alpha Alpha 66831 then press ENTER. |
| 3 | Press indicated key on the keypad. | SYS MODE MENU 1 EDIT PARAMATERS F2 DOWNLOAD F3 RESTART F4 | Press the purple key below to advance to MEMORY FUNCTIONS and select [F2] |
| 4 | Press indicated key on the keypad | SYS MODE MEM FUNCS RAM FLASH USAGE F2 DIRECTORIES F3 CLEAR MEM F4 | Press [F4] to clear the memory. |
| 5 | Press indicated key on the keypad | SYS MODE MEMORY ENTER GROUP 1 | Select [ENTER] if the GID displayed is correct. If not enter the correct GID and press [ENTER] |
| 6 | Press indicated key on the keypad | SYS MODE MEMORY CLEAR CONFIG SYS F2 CLEAR G1 FILES F3 CLEAR ALL GROUPS F4 | Select [F4] to clear all groups or [F3] to clear the single group. |
| 7 | Press indicated key on the keypad | SYS MODE MEMORY DELETE ALL GROUP RAM AND FLASH FILES CANCEL F3 CONFIRM F4 | Select [F4] to confirm |
| Note | The terminal will clear the RAM and FLASH memory and briefly flash a message indicating these files have been cleared. It will then go to the below screen. |  |  |
| 8 | Press indicated key on the keypad | SYS MODE MEM FUNCS RAM FLASH USAGE F2 DIRECTORIES F3 CLEAR MEM F4 | Select the red [CANCEL] key to return to the SYS MODE menu |
| 9 | Press indicated key on the keypad | SYS MODE MENU 2 MEMORY FUNCTIONS F2 TERMINAL INFO F3 CLOCK F4 | Press the purple arrow key to scroll to SYS MODE MENU 1 |
| 10 | Press indicated key on the keypad | SYS MODE MENU 1 EDIT PARAMETERS F2 DOWNLOAD F3 RESTART F4 | Select [F2] to edit parameters. |
| 11 | Press indicated key on the keypad | GROUP SELECT GROUP ID 1 APP | Press [ENTER] |
| 12 | Press indicated key on the keypad | SYS MODE FILE G1 FILE CONFIG SYS | Select [enter] |
| 13 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *GO VALUE EDIT F:IMM.OUT CLEAR | Use the purple scroll keys to scroll to the *ZP parameter. This should contain the download phone number. |
| 14 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | The download phone number should be displayed under the value. If it is not or it is incorrect select [F3] to edit and use the [BACKSPACE] key to clear and enter the information. |
| 15 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | Press the purple arrow key to scroll to the *ZT parameter. This should display the terminal id number. |
| 16 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | The terminal id number should be displayed under the value. If it is not or is incorrect select [F3] to edit. Use the [BACKSPACE] key to clear and enter the information. |
| 17 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | Press the purple arrow key to scroll to the *ZA application name. This should be the application name in the download file. |
| 18 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | The application name should be displayed under the value. If it is not or is incorrect select [F3] to edit. Use the [BACKSPACE] key to clear and enter the information. |
| 19 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | Press the red [CANCEL] key to return to the system mode menu. |
| 20 | Press indicated key on the keypad | SYS MODE MENU 1 EDIT PARAMATERS F2 DOWNLOAD F3 RESTART F3 | Select [F3] to begin the download. |
| 21 | Press indicated key on the keypad | SYSTEM MODE DOWNLOAD GROUP ID 1 | Press [ENTER] |
| 22 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 SINGLE APP F3 MULTI APP F4 | Select [F3] for a single application load. Note – Nova does not currently support F4 or multi application loads. |
| 23 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 FULL F3 PARTIAL F4 | Select [F3] for a full download. |
| 24 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 MODEM F2 COM 1 F3 TCPIP F4 | Select [F2] for a modem download. |
| 25 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 *ZA APPLICATION ID ----------------------------- | If prompted for the *ZA application name type in *MA |
| 26 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 *ZA = *ZP = EDIT F3 *ZR = *ZT = START F4 | On this screen the *ZA parameter should be the application name or *MA. *ZP should be the download phone number. *ZR should equal 6, and *ZT should be the terminal id number. If any of these values are missing or incorrect select [F3] to edit by using the backspace key and typing in the information. |
| 27 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 *ZA = *ZP = EDIT F3 *ZR = *ZT = START F4 | If all of the information on this screen is correct select [F4] to start the download. |
| 28 |  | SYS MODE DOWNLOAD G1 GID 1 APP ID *MA STATUS: CONNECTING | As the terminal connects the status will say connecting. |
| 29 |  | SYS MODE DOWNLOAD G1 GID 1 APP ID *MA STATUS: DOWNLOADING ------------------------ | As the terminal is downloading the status will say downloading with a series of dashes at the bottom. As the download progresses the dashes will turn to stars. This should take approximately 10 minutes. |
| 30 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 GID 1 APP ID *MA STATUS: DOWNLOADING DOWNLOAD DONE | When the download is complete the status will indicate that the download is done. At this point press [ENTER] |
| 31 | The terminal will re-boot and the download will be complete |  |  |

# PARTIAL DIAL DOWNLOAD

| Key | Value |
| --- | --- |
| *ZA | Application name |
| *ZT | Merchants NOVA number |
| *ZP | VeriCentre phone number |

| PARTIAL DIAL DOWNLOAD |  |  |  |
| --- | --- | --- | --- |
| Step | Action | Display | Keys |
| 1 | Press indicated key on the keypad. | <Current Menu Display> | Press [F2] + [F4] simultaneously. |
| 2 | Press indicated key on the keypad. | SYSTEM MODE ENTRY PASSWORD | Key in 1 Alpha Alpha 66831 then press ENTER. |
| 10 | Press indicated key on the keypad | SYS MODE MENU 1 EDIT PARAMETERS F2 DOWNLOAD F3 RESTART F4 | Select [F3] to edit parameters. |
| 11 | Press indicated key on the keypad | GROUP SELECT GROUP ID 1 APP | Press [ENTER] |
| 12 | Press indicated key on the keypad | SYS MODE FILE G1 FILE CONFIG SYS | Select [enter] |
| 13 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *GO VALUE EDIT F:IMM.OUT CLEAR | Use the purple scroll keys to scroll to the *ZP parameter. This should contain the download phone number. |
| 14 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | The download phone number should be displayed under the value. If it is not or it is incorrect select [F3] to edit and use the [BACKSPACE] key to clear and enter the information. |
| 15 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | Press the purple arrow key to scroll to the *ZT parameter. This should display the terminal id number. |
| 16 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZT VALUE EDIT CLEAR | The terminal id number should be displayed under the value. If it is not or is incorrect select [F3] to edit. Use the [BACKSPACE] key to clear and enter the information. |
| 17 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZP VALUE EDIT CLEAR | Press the purple arrow key to scroll to the *ZA application name. This should be the application name in the download file. |
| 18 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZA VALUE EDIT CLEAR | The application name should be displayed under the value. If it is not or is incorrect select [F3] to edit. Use the [BACKSPACE] key to clear and enter the information. |
| 19 | Press indicated key on the keypad | GID 1 NEW PARAMATER FIND *ZA VALUE EDIT CLEAR | Press the red [CANCEL] key to return to the system mode menu. |
| 20 | Press indicated key on the keypad | SYS MODE MENU 1 EDIT PARAMATERS F2 DOWNLOAD F3 RESTART F4 | Select [F3] to begin the download. |
| 21 | Press indicated key on the keypad | SYSTEM MODE DOWNLOAD GROUP ID 1 | Press [ENTER] |
| 22 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 SINGLE APP F3 MULTI APP F4 | Select [F3] for a single application load. Note – Nova does not currently support F4 or multi application loads. |
| 23 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 FULL F3 PARTIAL F4 | Select [F4] for a partial download. |
| 24 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 MODEM F2 COM 1 F3 TCPIP F4 | Select [F2] for a modem download. |
| 25 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 *ZA APPLICATION ID ----------------------------- | If prompted for the *ZA application name type in *MA |
| 26 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 *ZA = *ZP = EDIT F3 *ZR = *ZT = START F4 | On this screen the *ZA parameter should be the application name or *MA. *ZP should be the download phone number. *ZR should equal 6, and *ZT should be the terminal id number. If any of these values are missing or incorrect select [F3] to edit by using the backspace key and typing in the information. |
| 27 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 *ZA = *ZP = EDIT F3 *ZR = *ZT = START F4 | If all of the information on this screen is correct select [F4] to start the download. |
| 28 |  | SYS MODE DOWNLOAD G1 GID 1 APP ID *MA STATUS: CONNECTING | As the terminal connects the status will say connecting. |
| 29 |  | SYS MODE DOWNLOAD G1 GID 1 APP ID *MA STATUS: DOWNLOADING ------------------------ | As the terminal is downloading the status will say downloading with a series of dashes at the bottom. As the download progresses the dashes will turn to stars. This should take approximately 5 minutes. |
| 30 | Press indicated key on the keypad | SYS MODE DOWNLOAD G1 GID 1 APP ID *MA STATUS: DOWNLOADING DOWNLOAD DONE | When the download is complete the status will indicate that the download is done. At this point press [ENTER] |
| 31 | The terminal will re-boot and the download will be complete |  |  |