There are 3 steps to a T4100 Terminal download:

| Step | Action | Display | Keys |
| --- | --- | --- | --- |
| Edit the TermMaster File: |  |  |  |
| 1 | If the merchant wants the host batch # to print correctly on receipts for the first batch you must perform this function. If this is not a concern, you may skip this step. The batch # that prints on receipts will sync up with the host after the first settlement is completed. - Verify the merchant settled their batch and determine what the current and next batch #’s in host are. - Call in too MSP Support for the below step if needed. - Log into TM Suites and pull up the merchant’s download file. - Go into the ‘NOVA’ acquirer page and update the current batch and next batch info. so that it will match what is in the merchant’s terminal after the download. - Save |  |  |
| Configure the Terminal’s Download Host Information: |  |  |  |
| 2 | Press the HOME key to get to the maintenance menu screen that shows the Nova application and the IP Diagnostics | Financial; IP Diagnostics. | HOME |
| 2a | Press ‘FUNCTION’ | - Deployment Config - App. Maintenance - Terminal Maintenance - Utility Functions | FUNCTION |
| 2b | Press ‘1’ for Deployment Config. | - Terminal Config - Configure Hosts - Download Now - Fast load | 1 |
| 2c | Press `1’ for YES | IP from DHCP Server? 1. YES 2. NO | Press 1 for Dial or DHCP IP Communications Press 2 for Static IP Communications |
| 2d | Press 1 to enable daylight savings time Press 2 to disable daylight savings time | Daylight Savings 1. Enable 2. Disable | Press 1 to enable daylight savings time Press 2 to disable daylight savings time |
| 2e | Enter in the current time | Enter Time AM xx:xx:xx PM | After typing in the correct time Press the 2nd white key on the left to select AM or Press the 2nd white key on the right to select PM |
| 2f | Press 1 to configure the download Host | Cfg. Download Host? 1. Yes 2. No | 1 |
| 2g | Press ‘2’ to edit host if already exists Press ‘1’ to create host if it does not exist | - Edit/View Host - Add New Host - Rename Host - Delete Host | Press [1] NOTE: Nova TermMaster is set up so that the download info is always being passed to the primary download host in the terminal. Downloads attempted from the secondary download host setup in terminal do not pick up changes. |
| 2h | Input Nova # from Eclipse OR if you are downloading app manager only from the Test TM, use Terminal ID (41000000 through 41000009) OR press ‘ENTER’ if info. is already correct. | Terminal ID XXXXXXXX | Nova # or App Manager # NOTE: You do not need a check digit. |
| 2i | Select 1 if you will be performing a dial download Select 2 if you will be performing an IP download | - Dial - IP/Ethernet - RS-232 | 1 for Dial 2 for IP |
| 2j | Input the appropriate download parameters as requested by the terminal | Follow prompts on screen | For Dial: - Standard Dialup - Phone -1888-545-2230 (Input PBX if needed) - Security Level –None - Configure another host=No - Save Changes=Yes For IP: - IP Address-Production=198.203.192.20 Port 1405. Test= 198.203.191.87 Port 444 - Security-Production=SSL or Test=None, then SSL Gateway. - Configure another host=No - Save Changes=Yes |
| Download Now |  |  |  |
| 3a | Press the HOME key to get to the maintenance menu screen that shows the Nova application and the IP Diagnostics | Financial; IP Diagnostics. | HOME |
| 3b | Press FUNCTION | - Deployment Config - App. Maintenance - Terminal Maintenance - Utility Functions | FUNCTION |
| 3c | Press 1 for Deployment Config. | - Terminal Config - Configure Hosts - Download Now - Fast load | 1 |
| 3d | Press 3 to Download Now | This action may delete transactions. Settle now, Press Cancel to Exit. NOTE: A download on a Hypercom typically does not delete transactions; however, it may corrupt them, so it is best to settle them before performing a download. | 3 |
| 3e | Press ‘Enter’ NOTE: If the Screen does not show progression in %, please double check your parameters in configure hosts. If problem persists refer to Troubleshooting section for ‘Download Troubleshooting/ | <Connects using dial or IP params configured> Download In Progress Xxxx: Manifest.xxx Xxxx: Success | Press [ENTER] |
| 3f |  | Terminal Reboots Financial; IP Diagnostics. | Select [Financial] |