Hypercom T4200 Dial/IP Downloads Overview

The T4200 terminal application architecture looks much like a PC where applications have icons, and updating the application is much like an overwrite of what was there. As a result, there is not a parameter download, so the initialize key does not exist.

- The average dial download takes approximately 5-10 minutes

- The average IP download takes approximately 2-3 minutes

NOTE:  The above times are average. Download times can vary, and are highly dependent upon how many updates the terminal must make to become current.

Full / Partial Download Tips

| If you are attempting to perform the first Application IP download on a merchant owned T4200 (that Elavon did not deploy), you must perform the following steps first. |  |  |
| --- | --- | --- |
| Step | Action | Display |
| 1 | Press [HOME] | IP Diagnostics Financial |
|  | Press [FUNCTION] | - Deployment Config. - App. Maintenance - Terminal Maintenance - Utility Functions |
|  | Press [4] for Utility Functions | - Print Menu - Display Info - Terminal Config - Unload Applications |
|  | Press [2] to Display Info | - Display AppMgr. Ver - Display Xilinx Ver - Display Serial Number - Display Network Act |
|  | Press [1] to Display AppMgr. Ver | AM v X.X.X XXXXXXXX OS: XXXXXXXX Boot: XXXXXXXX |
| 2 | Press [HOME] to get to the Maintenance Menu screen | IP Diagnostics Financial |
|  | Press [FUNCTION] | Maintenance - Deployment Config - App. Maintenance - Terminal Maintenance - Utility Functions |
|  | Press [3] Terminal Maintenance | Terminal Maintenance - Terminal Settings - Configure Hosts |
|  | Press [2] for Configure Hosts | Download Host Menu - Edit/View Host - Add New Host - Add Firmware Host - Rename Host |
|  | Press [1] for Edit/View Host | Host Configuration - Host Terminal ID - Host Connection Type - Host Properties |
|  | Press [3] for Host Properties | Host Properties - Nll - Timeout - Retry Attempts - Rsp. Blk. Size - STID Check Digit - Server Security |
|  | Press [6] for Server Security | Host Security - Security Level - Security Connection NOTE: If you do not see a Security Level, you will not be able to download the terminal. Merchant has a terminal that has been ‘locked’ by their previous processor. |

Dial Download Instructions

| Step | Action | Display |
| --- | --- | --- |
| 1 | Press [HOME] to get to the Maintenance Menu screen | IP Diagnostics Financial |
| 2 | Press [FUNCTION] | Maintenance - Deployment Config - App. Maintenance - Terminal Maintenance - Utility Functions |
| 3 | Press [1] for Deployment Config | Deployment Config - Terminal Config - Configure Hosts - Download Now - Fast Load |
| 4 | Press [1] for Terminal Config | IP from DHCP Server? - YES - NO NOTE: Press [1] for Dial or DHCP IP Communications. Press [2] for Static IP Communications |
| 5 | Press [1] for YES | Daylight Savings - Enable - Disable |
| 6 | Press [1] to enable daylight savings time. or Press [2] to disable daylight savings time. | Set Terminal Time Enter time AM xx:xx:xx PM |
| 7 | Enter in the current time After typing in the correct time, choose the correct ‘hot key’ for AM or PM | Cfg. Download Host? - YES - NO |
| 8 | Press [1] to Configure the download host | Download Host Menu - Edit/View Host - Add New Host - Rename Host - Delete Host |
| 9 | Press [1] to Edit Host | Download Host Terminal ID Enter Terminal ID xxxxxxxx |
| 10 | Input Elavon # from Eclipse (NOTE: You do not need a check digit) or Press [ENTER] if information is already correct | Download Connection Type - Dial - IP/Ethernet - RS-232 |
| 11 | Select [1] for Dial | Dial Download Menu - Standard dialup - PPP dialup |
| 12 | Select [1] for Standard dialup | Download Phone Number Enter Number |
| 13 | Enter the Download Phone Number and press [ENTER] NOTE: If merchant’s location requires a PBX, place it in front of the download phone number | Security Level - None - SSL - SSL + Auth NOTE: Dial downloads do not need a Security Level set |
| 14 | Select [1] for None | Cfg. Another Download Host? - YES - NO |
| 15 | Select [2] for No | Save New Changes? - YES - NO |
| 16 | Select [1] for YES Saving Config. Data screen displays | Deployment Config - Terminal Config - Configure Hosts - Download Now - FastLoad |
| 17 | Select [3] for Download Now | Application Manager This action may Delete transactions. Settle now. Press Cancel to Exit |
| 18 | Press [ENTER] to continue | Application Manager Connecting Terminal will connect for download, reboot, and return to the Desktop |

IP Download Instructions

| Configure the Terminal’s download host information |  |  |
| --- | --- | --- |
| 1 | Press [HOME] to get to the Maintenance Menu screen | IP Diagnostics Financial |
| 2 | Press [FUNCTION] | Maintenance - Deployment Config - App Maintenance - Terminal Maintenance - Utility Functions |
| 3 | Press [1] for Deployment Config | Deployment Config - Terminal Config - Configure Hosts - Download Now - FastLoad |
| 4 | Press [1] for Terminal Config | IP from DHCP Server? - YES - NO NOTE: Press [1] for Dial or DHCP IP Communications. Press [2] for Static IP Communications |
| 5 | Press [1] for YES | Daylight Savings - Enable - Disable |
| 6 | Press [1] to enable daylight savings time. or Press [2] to disable daylight savings time. | Set Terminal Time Enter time AM xx:xx:xx PM |
| 7 | Enter in the current time After typing in the correct time, choose the correct ‘hot key’ for AM or PM | Cfg. Download Host? - YES - NO |
| 8 | Press [1] to Configure the download host | Download Host Menu - Edit/View Host - Add New Host - Rename Host - Delete Host |
| 9 | Press [1] to Edit Host | Download Host Terminal ID Enter Terminal ID xxxxxxxx |
| 10 | Input Elavon # from Eclipse (NOTE: You do not need a check digit) or press [ENTER] if information is already correct | Download Connection Type - Dial - IP/Ethernet - RS-232 |
| 11 | Select [2] for IP/Ethernet Download Connection Type | Host Connection Method? - IP Address - Host Name |
| 12 | Select [1] for IP Address | Download Host IP Address xxx.xxx.xxx.xxx |
| 13 | Enter the IP Address NOTE: Use the ‘*’ key to make the decimals | Download Host Port Number xxxx |
| 14 | Enter the Port Number | Security Level - None - SSL - SSL + Auth |
| 15 | Select [2] for SSL | Security Connection - HyperSSL - SSL Gateway |
| 16 | Select [1] for HyperSSL | Cfg. Another Download Host? - YES - NO |
| 17 | Select [2] for No | Save New Changes? - YES - NO |
| 18 | Select [1] for YES Saving Config Data screen displays | Deployment Config - Terminal Config - Configure Hosts - Download Now - FastLoad |
| 19 | Select [3] for Download Now | Application Manager This action may Delete transactions. Settle now. Press Cancel to Exit. |
| 20 | Press [ENTER] to continue | Terminal will connect for download, reboot, and return to the Desktop |